Firmware Design Document

Non-Blocking SPI DMA Driver with FreeRTOS Synchronization

Hardware Target: RP2350 / Raspberry Pi Pico 2

| Document Number | ESP-FW-001 |
| --- | --- |
| Revision | 1.0 |
| Status | RELEASE CANDIDATE — For Test & Production Approval |
| Date | October 13, 2026 |
| Author | Eric L. Johnson |
| Organization | Embedded Solutions Provider, LLC |
| Prepared For | [Client Name] |
| Classification | Confidential |

# 1. Document Approval

This document must be signed by the responsible parties before firmware is released to the Test phase and subsequently to Production.

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Firmware Author | Eric L. Johnson |  |  |
| Test Lead |  |  |  |
| Engineering Manager |  |  |  |
| Release Authority |  |  |  |

# 2. Overview and Purpose

This document describes the design, architecture, and interface of the Non-Blocking SPI DMA Driver (spi_dma_driver) developed for the RP2350 microcontroller platform using the Raspberry Pi Pico SDK and the FreeRTOS real-time operating system.

The purpose of this document is to serve as the authoritative technical reference for:

- Firmware engineering review and code sign-off

- Test team validation planning and test case development

- Production release gate approval

- Ongoing maintenance and future revision baseline

# 3. Hardware Platform and Dependencies

| Microcontroller | RP2350 (Raspberry Pi Pico 2) |
| --- | --- |
| SDK | Raspberry Pi Pico SDK (pico/stdlib.h, hardware/spi.h, hardware/dma.h) |
| RTOS | FreeRTOS (task.h, semphr.h) |
| SPI Interface | SPI0 hardware peripheral |
| DMA Controller | RP2350 integrated DMA, IRQ0 |
| Clock Speed | 1 MHz SPI baudrate (configurable) |
| GPIO Pins | SCK: GP2, TX/MOSI: GP3, RX/MISO: GP4, CS: GP5 |
| Transfer Width | 8-bit (DMA_SIZE_8) |

# 4. Architecture and Design Description

## 4.1 Design Philosophy

The driver implements a non-blocking, full-duplex SPI transfer mechanism using the RP2350 DMA controller. Rather than polling for transfer completion (which wastes CPU cycles) or using blocking delays, the driver uses a FreeRTOS binary semaphore to yield the calling task during the transfer and resume it when the DMA interrupt fires. This allows other RTOS tasks to execute during long SPI transfers.

## 4.2 Component Overview

| File | Description |
| --- | --- |
| spi_dma_driver.h | Public API header — configuration structure, status codes, function declarations |
| spi_dma_driver.c | Driver implementation — init, DMA config, ISR, transfer control |
| main.c | Test application — FreeRTOS task demonstrating driver usage |

## 4.3 Full System Sequence Diagram

Figure 1 shows the complete interaction between the FreeRTOS task, the driver API, the DMA controller, the interrupt service routine, and the binary semaphore during a single SPI transfer.

Figure 1 — Full System Sequence: FreeRTOS Task → Driver API → DMA → ISR → Semaphore

## 4.4 Transfer Sequence (Narrative)

The following sequence describes a single spi_dma_transfer_async() call from initiation to completion:

- 1. Caller provides TX buffer, RX buffer, length, and timeout in milliseconds.

- 2. Driver configures RX DMA channel: source = SPI DR register, destination = rx_buf, increment write pointer.

- 3. Driver configures TX DMA channel: source = tx_buf, destination = SPI DR register, increment read pointer.

- 4. Chip Select (GP5) is asserted LOW to begin the SPI transaction.

- 5. RX DMA channel is started first, then TX DMA channel (ensures no data is missed).

- 6. Calling FreeRTOS task blocks on xSemaphoreTake() with the specified timeout.

- 7. On RX DMA completion, spi_dma_isr() fires, clears DMA_IRQ_0, and calls xSemaphoreGiveFromISR().

- 8. Blocked task resumes; Chip Select is de-asserted HIGH; function returns SPI_DMA_OK.

- 9. On timeout, Chip Select is de-asserted and SPI_DMA_ERROR_TIMEOUT is returned.

## 4.5 DMA Channel Configuration Flow

Figure 2 shows the step-by-step logic inside spi_dma_transfer_async() for configuring both DMA channels before the transfer begins.

Figure 2 — DMA Channel Configuration Flow (spi_dma_transfer_async entry path)

## 4.6 Interrupt Service Routine

The ISR (spi_dma_isr) is registered as an exclusive handler on DMA_IRQ_0. It performs three operations:

- Clears the RX DMA channel interrupt flag in dma_hw->ints0.

- Releases the binary semaphore via xSemaphoreGiveFromISR().

- Triggers a context switch via portYIELD_FROM_ISR() if a higher-priority task was unblocked.

## 4.7 FreeRTOS Semaphore Block / Unblock Sequence

Figure 3 shows how the FreeRTOS binary semaphore coordinates the task block during the DMA transfer and the unblock triggered by the ISR, including the timeout path.

Figure 3 — FreeRTOS Semaphore Block/Unblock: Normal completion and timeout paths

## 4.8 Null Buffer Support

Both TX and RX buffers are optional. If tx_buf is NULL, a static dummy byte (0xFF) is transmitted. If rx_buf is NULL, received bytes are discarded into a static dummy variable. This supports TX-only and RX-only modes without requiring the caller to allocate unused buffers.

# 5. API Reference

## 5.1 Status Codes

| Status Code | Description |
| --- | --- |
| SPI_DMA_OK | Operation completed successfully |
| SPI_DMA_ERROR_INVALID_PARAM | NULL pointer or zero-length transfer passed |
| SPI_DMA_ERROR_BUSY | Driver busy (reserved for future use) |
| SPI_DMA_ERROR_TIMEOUT | Transfer did not complete within timeout_ms |
| SPI_DMA_ERROR_HARDWARE | FreeRTOS semaphore creation failed at init |

## 5.2 Configuration Structure — spi_dma_config_t

| Field | Description |
| --- | --- |
| spi_instance | Pointer to SPI hardware peripheral — spi0 or spi1 |
| baudrate_hz | SPI clock frequency in Hz (e.g., 1000000 for 1 MHz) |
| pin_sck | GPIO pin number for Serial Clock |
| pin_tx | GPIO pin number for MOSI (Master Out Slave In) |
| pin_rx | GPIO pin number for MISO (Master In Slave Out) |
| pin_cs | GPIO pin number for Chip Select (active LOW) |

## 5.3 spi_dma_init()

spi_dma_status_t spi_dma_init(const spi_dma_config_t *config);

Initializes the SPI peripheral, GPIO pins, DMA channels, FreeRTOS semaphore, and DMA interrupt handler. Must be called once before any transfers. Returns SPI_DMA_OK on success.

| Parameter | Description |
| --- | --- |
| config | Pointer to a populated spi_dma_config_t structure. Must not be NULL. |
| Returns | SPI_DMA_OK, SPI_DMA_ERROR_INVALID_PARAM, or SPI_DMA_ERROR_HARDWARE |

## 5.4 spi_dma_transfer_async()

spi_dma_status_t spi_dma_transfer_async(const uint8_t *tx_buf, uint8_t *rx_buf, size_t len, uint32_t timeout_ms);

Executes a non-blocking full-duplex SPI transfer using DMA. The calling FreeRTOS task yields until the transfer completes or the timeout expires. Must not be called from an ISR context.

| Parameter | Description |
| --- | --- |
| tx_buf | Pointer to transmit data. Pass NULL for RX-only transfers. |
| rx_buf | Pointer to receive buffer. Pass NULL for TX-only transfers. |
| len | Number of bytes to transfer. Must be greater than 0. |
| timeout_ms | Maximum wait time in milliseconds before returning TIMEOUT. |
| Returns | SPI_DMA_OK or SPI_DMA_ERROR_TIMEOUT or SPI_DMA_ERROR_INVALID_PARAM |

# 6. Known Limitations and Open Items

| Item | Description |
| --- | --- |
| Single instance only | The driver uses a single static control block. Only one SPI DMA instance can be active per build. |
| CS pin hardcoded in transfer | Chip Select (GP5) is referenced by literal value in spi_dma_transfer_async(). Should use config struct value in future revision. |
| No re-entrancy protection | Concurrent calls to spi_dma_transfer_async() from multiple tasks are not protected. Caller must ensure mutual exclusion. |
| DMA_ERROR_BUSY not implemented | SPI_DMA_ERROR_BUSY is defined but not returned. Busy detection is a future enhancement. |
| No deinit function | No spi_dma_deinit() is provided. DMA channels and IRQ handler persist until system reset. |

# 7. Test Criteria and Pass/Fail Conditions

The following tests must pass before firmware is approved for production release. The Test Lead is responsible for executing and documenting each test result.

| ID | Test Case | Pass Condition | Result |
| --- | --- | --- | --- |
| T01 | Driver Initialization | spi_dma_init() returns SPI_DMA_OK with valid config |  |
| T02 | Null Config Rejection | spi_dma_init(NULL) returns SPI_DMA_ERROR_INVALID_PARAM |  |
| T03 | Full-Duplex Transfer | 64-byte TX pattern received correctly on RX loopback |  |
| T04 | TX-Only Transfer | tx_buf valid, rx_buf NULL — completes without fault |  |
| T05 | RX-Only Transfer | tx_buf NULL, rx_buf valid — completes without fault |  |
| T06 | Timeout Detection | Transfer with no slave returns SPI_DMA_ERROR_TIMEOUT |  |
| T07 | CS Assertion Timing | CS asserted LOW before first byte, HIGH after last byte (scope verify) |  |
| T08 | FreeRTOS Task Yield | Other tasks execute during transfer (verified by task log) |  |
| T09 | ISR Context Switch | portYIELD_FROM_ISR triggers correct task switch on completion |  |
| T10 | Repeat Transfers | 1000 consecutive transfers complete without error or hang |  |

# 8. Revision History

| Revision | Description |
| --- | --- |
| 1.0 | Initial release — Non-blocking SPI DMA driver with FreeRTOS semaphore synchronization. RP2350 target. Three embedded architecture diagrams added. |

# 9. Release Gate Checklist

The following items must be completed and checked before firmware proceeds through each gate.

## 9.1 Design Review → Test Gate

- ☐  Design document reviewed and approved by Engineering Manager

- ☐  All Known Limitations in Section 6 reviewed and accepted or assigned

- ☐  Source code reviewed by second firmware engineer

- ☐  All files checked into version control with tag ESP-FW-001-RC1

## 9.2 Test → Production Gate

- ☐  All T01 through T10 test cases passed and documented

- ☐  Test Lead signature obtained on approval table (Section 1)

- ☐  No open P1 or P2 defects

- ☐  Binary build hash recorded for traceability

## 9.3 Production → Shipment Gate

- ☐  Release Authority signature obtained on approval table (Section 1)

- ☐  Build artifact archived per company configuration management procedure

- ☐  Customer delivery package prepared and reviewed